1	1	WEBX20	iQ-WEBX 20 Image Archive with Integrated Web Distribution

High Performance Web-Based DICOM Image Server

        - Multi-vendor operability to receive DICOM images from any attached DICOM 3.0 compliant modality

        - Accommodates simultaneous DICOM associations from up to 20 DICOM networked devices

        - Configurable automatic routing of studies to one or more specified workstations, archivers or other DICOM devices for viewing, evaluation and storage

        - Microsoft Windows® 2008 Server Operating System

        - Powerful Apache 2.2 Web Server with MySQL Database

        - Available Image Storage: Approximately 1.7 TB (5x1000); RAID 5 configuration

        - Bi-directional DICOM communication, iQ-X ActiveX viewer plug-in

        - Site configurable to permit automatic image forwarding by source or target addresses

        - Allows simultaneous, unlimited web-based image query, retrieve and display

        - Support of Microsoft Windows web users using Microsoft Internet Explorer version 6 through version 8

        - Web users work in familiar iQ-VIEW Workstation-like environment for efficient image review

        - Multi-level security limits access to authorized users with valid Username, Password and defined Privileges

        - Definable physician privileges restricts referring physicians to see only their own patients

        - Web users may print images to any compatible local or networked paper printer

        - Web Users have a wide range of image manipulation:

        - Window and level

        - Image zoom and pan

        - Image rotation, mirror with cine mode

        - Distance measurement, angle measurement, multiple image display

        - Scout view

        - Connectivity via 10/100/1000 BaseT Ethernet

        - Remote service link for off-site diagnosis and repair

        - The iQ-WEBX 20 Image Server includes:

        - Intel Dual Core Xeon Microprocessor

        - 1.7 Terabyte RAID5 Image Storage; Hot-swap Hard Drives

        - 4GB RAM, 100/1000 Network Connectivity

        - Dell Universal Rapid Rails for insertion in existing server rack

        - Microsoft Windows 2008® Server Operating System

2	1	iQ-D3MP	iQ-VIEW Dual-Display Diagnostic Workstation

FDA-Certified DICOM Compliant Workstation for Direct Interpretation

        - Simple, intuitive mouse-driven interface enhances productivity and promotes wide acceptance among medical staff

        - Based on the Intel Core Duo Microprocessor and Microsoft Windows® XP Professional Operating System

        - Supplied with two (2) IMAGE SYSTEMS 3 Megapixel Grayscale Professional Series dual Flat Panel LCD displays with nVIDIA PCIe Display Controller

        - Embedded Photometer provides consistant luminosity and easy calibration

        - System can receive studies from unlimited networked DICOM 3.0 compliant devices for subsequent image review

        - Query and retrieve images from other DICOM servers

        - Workstation can import and display DICOM images from CD’s from outside institutions when inserted into Workstation and permits storing imported images into iQ-WEBX Archive as part of patient’s permanent record

        - A full complement of tools provides a range of image manipulation:

        - Simultaneously view multiple views or multiple patient studies for easy comparison

        - Cross Reference line display for aid in review of cross-sectional imaging

        - Image pan, scroll, global zoom, window/level, rotation, ciné, B/W invert

        - Magnifying Glass facilitates small part visibility

        - Operator-selectable display matrices for cross sectional imaging allows for optimum CT or MR image display

        - Distance Measurement, Angle Measurement, Cobb Angle Calculations, Text Annotation with arrows

        - Annotated images may be saved to iQ-WEBX Archive with patient’s permanent record for subsequent viewing on iQ Web or iQ-VIEW Workstation

        - DICOM Print selected images or entire patient studies to DICOM-compliant film-based Laser Imagers or print images to Windows-compatible paper printers

        - Fully FDA, CE and DICOM Compliant

        - The iQ-VIEW Diagnostic Workstation consists of:

        - Microsoft Windows 7 Professional Operating System

        - 2GB RAM, 250 GB disk capacity

        - 10/100/1000 BaseT Network Connectivity

        - Dual 3-Megapixel High-Intensity Grayscale Displays

        - UPS with Automatic Shutdown Software

        - Integrated CD/DVD recorder, Extended Keyboard, mouse

3	1	IQ-3D	iQ-VIEW 3-D Module

Advanced 3-D Rendering Module for iQ-VIEW Diagnostic Workstation, to include:

- Orthogonal and Oblique Multiplanar Reconstruction (MPR)

- Maximum and Minimum Intensity Projections (MIP)

- Surface Shaded Display (SSD) and Volume Rendering (VRT)

- Integrated on iQ-VIEW Diagnostic Workstation

4	1	iQ-ADMIN	iQ-VIEW Administrative Workstation

FDA-Certified DICOM Compliant Workstation for Technologist Use

        - Simple, intuitive mouse-driven interface enhances productivity and promotes wide acceptance among medical staff

        - Based on the Intel Core Duo Microprocessor and Microsoft Windows® XP Professional Operating System

        - Supplied with one (1) 24” Widescreen Landscape DVI Color Display

        - System can receive studies from unlimited networked DICOM 3.0 compliant devices for subsequent image review

        - Query and retrieve images from other DICOM servers

        - Create fully DICOM Part 10-compliant patient CD’s, with single- or multiple-studies, with viewer deposited on each disk

        - CD’s may be reviewed on any computer running Microsoft Windows 2000/XP/Vista/7

        - CD’s display Institution Logo and contact information at top of patient list for a professional appearance when inserted into a computer for review

        - Workstation can import and display DICOM images from CD’s from outside institutions when inserted into Workstation and permits storing imported images into iQ-WEBX Archive as part of patient’s permanent record

        - A full complement of tools provides a range of image manipulation:

        - Simultaneously view multiple views or multiple patient studies for easy comparison

        - Cross Reference line display for aid in review of cross-sectional imaging

        - Image pan, scroll, global zoom, window/level, rotation, ciné, B/W invert

        - Magnifying Glass facilitates small part visibility

        - Operator-selectable display matrices for cross sectional imaging allows for optimum CT or MR image display

        - Distance Measurement, Angle Measurement, Cobb Angle Calculations, Text Annotation with arrows

        - Annotated images may be saved to iQ-WEBX Archive with patient’s permanent record for subsequent viewing on iQ Web or iQ-VIEW Workstation

        - DICOM Print selected images or entire patient studies to DICOM-compliant film-based Laser Imagers or print images to Windows-compatible paper printers

        - Fully FDA, CE and DICOM Compliant

        - The iQ-VIEW Administrative Workstation consists of:

        - Microsoft Windows 7 Professional Operating System

        - 2GB RAM, 250 GB disk capacity

        - 10/100/1000 BaseT Network Connectivity

        - 24” Widescreen Landscape DVI Color Display

        - UPS with Automatic Shutdown Software

        - Integrated CD/DVD recorder, Extended Keyboard, mouse

5	1	DCMR	DICOMREADER

Automated CD/DVD Image Importation

        - Provides a convenient method of importing DICOM studies from disks patients bring in from outside institutions

        - During importation, patient ID numbers may be changed to correspond to your institution’s unique patient ID regimen

        - Furnished installed and integrated on iQ-VIEW Administrative Workstation

6	1	IMP-10004	Imprinted Certified Media

        - Imprinted Certified CD Media for Patient Disk creation, Qty 100

CONTRACT TOTAL:	$ 00,000.00

NOTES ON THIS PROPOSAL:

AVAILABILITY:

Equipment appearing herein is available for shipment and installation approximately 04-06 weeks after receipt of order. Due to continual hardware and software development, stated specifications are subject to change without notice.

WARRANTY AND SUPPORT:

Server Software and Hardware:

- Server software support and upgrades are included for a period of twelve (12) months from first use.

- Server Hardware support is included for a period of thirty-six (36) months parts and labor.

Workstation Software and Hardware:

- Support and upgrades for supplied Software are included for a period of twelve (12) months from first use.

- 3-Megapixel Diagnostic Displays and supplied nVidia Graphics Card are covered for thirty-six (36) months parts.

- Dell-branded Hardware support is included for a period of thirty-six (36) months parts and labor.

To effect the above Warranties, Purchaser/End-user agrees to allow broadband internet access as specified by Precision for technical support. Failures or erratic system performance due to customer installation of unapproved software on devices appearing herein are expressly excluded from warranty consideration. Hardware support on Dell-branded Servers, RAID Arrays and Workstations is furnished by Dell ProSupport field service with same day 4-hour on-site response (24x7) for servers, NBD for workstations. Note the Dell 36-month warranty commences on the ship date from Dell manufacturing and will not coincide with actual system installation date. Customer may have to perform initial troubleshooting for Dell engineers. Items not specified above are warranteed for a period of 12-months parts and labor from first use. General release upgrades to furnished Application Software for all supplied items are included for a period of 12 months from installation at no additional cost.

FREIGHT, INSTALLATION AND SYSTEM APPLICATIONS:

Freight, Installation and System Applications are included with this Contract.

If this system requires connection to DICOM modalities, service representatives for those modalities must be present to set-up the DICOM parameters at the time of installation of this equipment. This configuration assumes all imaging devices are DICOM 3.0 compliant. On-site system applications are included with this Contract.

SITE PREPARATION:

The costs for items such as permits, engineering, electrical, structural, carpentry, heating/ventilating, wiring, hubs, routers, switches, assignment of IP addresses and any other items of networking for local- and/or wide-area communication and all other construction costs related to site preparation to permit installation, startup, proper function, networking and use of items quoted herein is the responsibility of the purchaser.